# Direct Material Costs

| Subsystem | Key Components | Function | Cost [€] |
| --- | --- | --- | --- |
| Processing & Control | MCU, Flash Memory | System control & communication | 5 |
| Measurement Chain | ADC, Amplifiers, Filters | Precision force measurement | 15 |
| HMI | Touchscreen, LEDs, Buttons | User interaction & status indication | 30 |
| Power System | Battery, Regulators | Portable power supply | 10 |
| PCB & Passives | PCB, resistors, capacitors | Electrical integration | 14 |
| Protective Case | Pelican Case | Structural Housing | 220 |
| Miscellaneous Mechanical Components | Screws, fasteners, glue, O-rings | Mechanical securing and sealing | 10 |
| Load Cells | 6 Innovatest load cells | Force measurement | 2844 |
| Total Direct Material Cost: |  |  | 3148 |

# Raw Material Costs

| Material | Quantity [kg] | Cost per kg [€/kg] | Cost [€] |
| --- | --- | --- | --- |
| Polycarbonate | 0.12 | 3.5 | 0.42 |
| ABS | 0.08 | 2.5 | 0.2 |
| 5052 Aluminum | 6.6 | 4 | 26.4 |
| EVA/PU Foam | 0.96 | 2 | 1.92 |
| Total Raw Material Cost: |  |  | 28.94 |

# 7. Material Overhead Costs

Material Overhead Rate = 5%

Material Overhead Costs = 5% * (Direct Material Costs + Raw Material Costs)

= 5% * (€3148 + €28.94) = €158.85

Therefore, the total material costs = €3148 + €28.94 + €158.85 = €3335.79/unit

# 8. Manufacturing Labor Costs

Assuming an hourly wage of €20/h, the labour costs can be calculated using the total working hours found in :

Total Annual Labor Costs = 4284h/year * €20/h = €85 680/year

Unit Labor Cost = €85 680/year ÷150 unit/year = €571.20/unit

# 9. Production Overheads

## 9.1 Machine Depreciation

To calculate the machine costs, the annual depreciation rate had to be considered to find the depreciation per year. Then, considering that the machines availability as found in Table , the machine hourly rates were found. Finally, the required machine hours per unit from Table, we used to calculate the machinery costs per unit.

| Machine | Total Cost [€] | Annual Depreciation Rate | Depreciation Per Year [€/year] | Machine Hourly Rate [€/h] | Machine Hours per Unit [h/unit] | Machine Cost per Unit [€/unit] |
| --- | --- | --- | --- | --- | --- | --- |
| 3D Printer | 800 | 20% | 160 | 0.1539 | 6.7 | 1.03 |
| 4-Axis CNC Milling Machine | 70000 | 10% | 7000 | 5.2910 | 2.72 | 14.37 |
| 3-Axis CNC Contour Machine | 50000 | 10% | 5000 | 4.0700 | 2.22 | 9.02 |
| 3-Axis CNC Milling Machine | 9000 | 10% | 900 | 0.6803 | 1.58 | 1.077 |
| Extrusion Press | 100000 | 10% | 10000 | 5.8789 | 0.28 | 1.67 |
| Drill | 150 | 10% | 15 | 0.0132 | 0.23 | 0.00309 |
| Reamer | 30 | 10% | 3 | 0.0032 | 0.27 | 0.0008466 |
| Laser Cutter | 40000 | 10% | 4000 | 3.0234 | 0.42 | 1.260 |
| Welding Plant | 500 | 10% | 50 | 0.0529 | 1.28 | 0.0679 |
| Automated Pick-and-Place Machine | 3500 | 10% | 350 | 0.2469 | 0.18 | 0.04527 |
| Reflow Oven | 360 | 5% | 18 | 0.0112 | 0.34 | 0.00383 |
| Hand Soldering Station | 200 | 5% | 10 | 0.0151 | 0.42 | 0.00630 |
| Total |  |  |  |  |  | 28.56 |

## 9.2 Utilities

| Machine | Power [kW] | Cost per kWh [€/kWh] | Machine Hours per Unit [h/unit] | Utility Cost per Unit [€/unit] |
| --- | --- | --- | --- | --- |
| 3D Printer | 0.15 | 0.04 | 6.70 | 0.04 |
| 4-Axis CNC Milling Machine | 8 | 2.1 | 2.72 | 45.64 |
| 3-Axis CNC Contour Machine | 2.7 | 2 | 2.22 | 11.97 |
| 3-Axis CNC Milling Machine | 1 | 2 | 1.58 | 3.17 |
| Extrusion Press | 10 | 0.8 | 0.28 | 2.27 |
| Drill | 0.05 | 0.02 | 0.23 | 0.00023 |
| Reamer | 0 | 0 | 0.27 | 0 |
| Laser Cutter | 4.5 | 1.17 | 0.42 | 2.19 |
| Welding Plant | 2.5 | 0.65 | 1.28 | 2.09 |
| Automated Pick-and-Place Machine | 0.2 | 0.05 | 0.18 | 0.00183 |
| Reflow Oven | 1.2 | 0.31 | 0.34 | 0.13 |
| Hand Soldering Station | 0.07 | 0.02 | 0.42 | 0.00058 |
| Total |  |  |  | 67.49 |

## 9.3 Factory Rent Allocation

Based on the requirements the facility size needed is estimated to be around 100 square meters and estimated rent is 7 euros per square meters.

Monthly Base Rent: 100m^2*€ 7/m^2 = €700/month

Yearly Base Rent: €700/month * 12 month/year = €8400/year

Additional Costs = €8400/year * 15% = €1260/year

Total Annual Rent = €8400/year +  €1260/year = €9660/year

Rent per Unit = €9660/year ÷ 150 unit/year = €64.40/unit

## 9.4 Total Production Overhead

Therefore our total manufacturing costs  =

= €571.20/unit + €28.56/unit + €67.49/unit + €64.40/unit = €731.65/unit

# 10. Total Production Costs

Therefore the total production costs:

€3335.79/unit + €731.65/unit = €4067.44/unit

While this is slightly above the budget outlined in Section, the main contributor to costs is the labour costs. If manufacturing of some parts are outsourced, this can potentially reduce the costs sufficiently enough to stay within the allocated €4000/unit budget.

# 11. Overheads

To account for administrative and marketing and sales overhead, we use simple margin percentages of 8% and 15% respectively.

Admin Overhead = 8% * €4067.44/unit = €325.39/unit

Marketing overhead = 15% * €4067.44/unit = €610.12/unit

Therefore,

COGS = €4067.44/unit +  €325.39/unit + €610.12/unit = €5002.95/unit

Finally, to calculate a reasonable offer price, we can use a 25% profit margin

Proft = 25% * €5002.95/unit = €1500.88/unit

Cash Selling Price = €5002.95/unit + €1500.88/unit = €6503.83/unit

With a distributor commission of around 10%,

Commission = 10% * €6503.83/unit = €650.38/unit

Therefore,

FINAL OFFER PRICE = €7154.21/unit

# 14. Summary

A target profit margin is applied to the cost of goods sold to determine the final selling price. This margin ensures sustainability while maintaining market competitiveness.

| Item | Value |
| --- | --- |
| Material Costs | €3335.79 |
| Manufacturing Costs | €731.65 |
| Production Costs | €4067.44 |
| COGS | €5002.95 |
| Cash Selling Price | €6503.83 |
| Proft | €1500.88 |
| Final Offer Price | €7154.21 |